Chevy LS Serpentine Front Drive Kit Install Instructions

| Box | Description |
| --- | --- |
| 1 | Universal AC Compressor w/ Pulley, Fitting & Hardwares |
| 2 | Universal Alternator w/ Pulley & Hardwares |
| 3 | Universal Belt Tensioner & Hardwares |
| 4 | Universal Idler Pulley & Hardwares |
| 5 | Universal GM Type II Power Steering Pump w/ Reservoir, Pulley & Hardwares |
| 6 | Chevy LS Crank Pulley Spacer & Hardwares |
| 7 | Chevy LS Crank Pulley & Hardwares |
| 8 | Chevy LS Water Pump w/ Pulley, Spacers & Hardwares |
| 9 | Chevy LS Main Bracket |
| 10 | Chevy LS 8 Rib Belt - 2100mm |

Chevy LS Serpentine Front Drive Kit Install Instructions

| Hardwares Pack List |  |  |  |
| --- | --- | --- | --- |
| Pack | Description | Comments | Qty |
| 1 | M8*25 SHCS | Bolts for AC Compressor | 2 |
| 2 | M8*25 SHCS | Bottom Alternator Bolt | 1 |
|  | M8*65 SHCS | Top Alternator Bolt | 1 |
|  | M8 Cap Nut | Nut for Top Alternator Bolt | 1 |
| 3 | M8*150 SHCS | Bolts for Water Pump to Bracket | 6 |
|  | 63mm Drivepipe | Drivepipes for Water Pump | 6 |
|  | Sealing Gasket | Sealing Gaskets for Water Pump | 2 |
| 4 | 3/8-16*3" Hexagon Bolt | Bolt for Belt Tensioner | 1 |
|  | 3/8-16 Self-locking Nut | Nut for Belt Tensioner Bolt | 1 |
|  | 3/8 Plain Washer | Washer for Belt Tensioner Bolt | 1 |
| 5 | 3/8-16*3.25" SHCS | Bolts for Crank Spacer | 3 |
|  | M16*150 Hexagon Bolt | Bolt for Crank Spacer | 1 |
|  | M16 Plain Washer | Washer for Crank Spacer Bolt | 1 |
|  | M16 Spring Washer | Washer for Crank Spacer Bolt | 1 |
| 6 | 3/8-16*1"SHCS | Bolts for Crank Pulley | 3 |
| 7 | M8*30 SHCS | Bolts for Power Steering Pump | 2 |